## D Flip-Flop Simulations      Partner -> Sam A.	answers for D Flip-Flop Simulations lab

round all your times to the nearest picosecond

| ### 1. D Flip-Flop clk-to-q Time | ### 3. D Flip-Flop Hold Time |
| --- | --- |
| Baseline CLK-TO-Q Times Simulation 1 data high CLK-to-Q = 569 ps Simulation 1 data low CLK-to-Q = 504 ps | Simulation 3 data high hold time = -50 ps CLK-to-Q delay = 577 ps Simulation 3 data low hold time = -70 ps CLK-to-Q delay = 510 ps |
| ### 2. D Flip-Flop Setup Time | ### 4. D Flip-Flop Data Requirements |
| Simulation 2 data high setup = 220 ps CLK-to-Q delay = 578 ps Simulation 2 data low setup = 260 ps CLK-to-Q = 513 ps | Simulation 4 data high 11.73 ns to 12.1 ns CLK-to-Q delay = 570 ps Simulation 4 data low 19.79 ns to 20.05 ns CLK-to-Q delay = 513 ps |